Notes for BCB Class of 1/10/17

When underwriting trade contractors, ask the following questions:

Who?  What? Where? When? Why? How?

Understanding who they are, what they do and how they do it (scale and scope of operations) helps us to quantify exposure and understand what controls should be in place

Who: Who are we covering? How many years of experience?  Do they have any Certifications?  Do they have any Licensing?

What: What kind of work or operations are they performing? Can we quantify the size and scope of the work? How specialized is the work? Do they have experience with this type of operation?

Where: Two perspectives-the actual location they are working at-shop, job site? Are there specific exposures? Venue-what are the implications? How litigious is the environment-labor law-third party action over?

When: Time of day and/or length of job can influence performance?

Why and/or How: Who are they working for?  What are the contractual obligations?  Is there proper risk transfer in place?

Carpentry Contractors

Prem Ops Hazard Grade = 7

PCO Hazard Grade = 6

Exposure: 3rd party PD, BI and CD

Framing Contractors:

(aka rough carpentry)

Lay out and erect structural skeletons; build forms for concrete construction including foundations

Finishing Contractors:

Interior work on standing buildings; fitting of doors and windows, trim, paneling, flooring, cabinets, staircases, ornamental woodwork

Some carpenters may work solely out of a shop but most will have some amount of construction site exposure.

Typically, an individual is required to apprentice 4-5 years to learn the trade.  Certifications and Association memberships are signs of experience and expertise.

Prem Ops:

BI/PD at job sites are primary exposures-may be some visitor experience but mostly centered around workers.  The actual work site can present a host of dangers to workers.  S/F and T/F are common- sawing, sanding, woodcutting, scaffolding-fall from height, struck by falling object-all are potential causes of injury or damage.

PCO:

BI/PD arising out of poor workmanship; failure to meet the specifications, building codes or allegations of sub-standard materials are primary exposures.

New construction-possibility of CD/poor materials.

Repair/Remodel-retrofitting can be difficult-can result in damage to surrounding area

Plumbing Contractors

Prem Ops Hazard Grade = 7

PCO Hazard Grade = 6

Exposure: Potential injuries from digging operations-damage to customer’s property

Installation, fitting, repairing and maintenance of the water, waste disposal, drainage and gas systems in residential, commercial and industrial buildings.

Typically, work on bathroom fixtures, piping for appliances-automatic sprinkler systems, fire lines and stand pipes used for fire protection, irrigation systems, septic systems-may also perform excavating services.

Like carpenters, apprentice training and licensing requirements

Prem Ops:

Careless or incorrect use of equipment can cause excessive noise, PD and/or BI to customers

PCO:

Faulty and incorrect waste and vent pipe installation and maintenance-could result in fires, toxic gases, flooding and water damage. Pressure vessel welding can result in leaks and explosions. Installation of sprinkler systems and drains can result flooding and water damage.

HVAC

Prem Ops Hazard Grade = 6

PCO Hazard Grade = 7

Exposure: BI and PD arising out of operations

Install, repair and maintain mechanical, electrical and electronic components that make up heating and air conditioning units in residential, commercial and industrial buildings

Work with motors, compressors, pumps, fans, ducts, pipes, thermostats and switches-may also sell furnaces, air conditioning units, thermostats, etc so there is a products exposure as well

Roofing Contractors

Prem Ops Hazard Grade = 7

PCO Hazard Grade = 5

Exposure: BI and PD arising out of operations-water leaks/damage and fires.

Repair, install roofs-wide variety of materials can be used-asphalt, tar, gravel, rubber, thermoplastic, metal, slate, fiberglass, wood or tile.

Some roofers may also perform waterproofing, siding, gutters, solar panels, windows and doors-building envelope work should be carefully reviewed.

Most residential work uses asphalt while commercial and industrial risks typically implement single ply sheets of synthetic rubber or asphalt; synthetic compounds of multi-ply built layers of membrane materials. Most roofers will specialize in one type of roofing-residential or commercial/industrial.

Training is typically on the job and there is a high percentage of seasonal/temporary/unskilled workers.  This class has a higher than average employee turnover rate due to occupational illness and injury-this is driven by a large percentage of part-time/transient workers.

Concrete/Masonry Contractors

Prem Ops Hazard Grade = 7

PCO Hazard Grade = 7

Exposure: 3rd party BI and PD-collapse, leaks and water damage.

Trained craftsmen in stone setting, bricklaying, marble setting, construction using concrete tiles or blocks.  They may build walls, floors, building partitions, veneer exteriors, walkways, walls patios.  Some may also perform excavation operations which involves grading and land preparation as well as pour foundations for roads, houses or other buildings.

Resources

AM Best Underwriting Guides

Loss Control-Lexis/Nexis Reports

OSHA-Check for Violations and Fines

WC Experience-Check Experience Mods-good indicator of management

Google

Customer Feedback/Referral Sites-Angies list, Home Advisor, Yelp, BBB, Local Associations

Positive Characteristics to Look For

Specified Operations

Years In Business

Similar Experience

Certifications

Association Memberships

Formal Safety Protocols

Formalized Recall Plan